101237 Final

HQ and Branch User Network Information:

1.     HQ user 1

a.     IP Network: 172.26.101.0 /24

b.     IP assigned by DHCP

c.     VLAN: 101 (HR)

2.     HQ user 2

a.     IP Network: 172.26.201.0 /24

b.     IP assigned by DHCP

c.     VLAN: 201 (Sales)

3.     Branch User 1

a.     IP Network: 192.168.11.0 /24

b.     IP assigned by DHCP

c.     VLAN: 11 (R&D)

Basic Network Requirements:

1.     Trunks

a.     Native VLAN: 99

b.     Interfaces should be hard coded to be a trunk.

c.     DTP should be disabled and not negotiated

2.     Access Ports

a.     Hard coded to be an access port

b.     Configured with correct vlan

c.     Port Security is enabled

•     Maximum MAC Addresses: 3

•     Violation: Protect

•     MAC Address Security: Sticky

d.     Portfast: Enabled

e.     BPDUGuard: Enabled

3.     VLANs

a.     HQ_S1

i     VLAN 101 Name: HR

•     Assigned Port: Fa0/1

ii     VLAN 50 Name: MGMT

iii     VLAN 99 Name: native

b.     HQ_S2

i.     VLAN 201 Name: Sales

•     Assigned Port: Fa0/1

ii     VLAN 50 Name: MGMT

iii     VLAN 99 Name: native

c.     Branch_R1

i.     VLAN 11 Name: R&D

ii     VLAN 99 Name: native

d.     Branch_S1

i.     VLAN 11 Name: R&D

•     Assigned Port: Fa0/1

ii     VLAN 99 Name: native

4.     Hostnames

a.     All devices, including ISP and all other routers, must have a hostname configured. Host names can be found in addressing table in the device column

Note:     Hostnames are case sensitive and must match the hostname in the addressing table

Routing

1:   IP Addressing

a.     Configure IP addresses on interfaces per IP address table

b.     You will need to configure an IP address directly on Branch_R1 interface Gig 0/2

2:   Static Routes

a.     Configure one static route on the internal network so any device on the internal network can route to any IP address on the internet.

b.     Configure one static route on the ISP router so the ISP router can route to any internal IP address

3:   OSPF Requirements

a.     Process ID = 10

b.     Each router should have a unique router identifier per below list

•     Edge: 1.1.1.1

•     HQ_R1: 2.2.2.2

•     HQ_R2: 3.3.3.3

•     Branch_R1: 4.4.4.4

c.     Only router interfaces that are connected to another device participating in OSPF should send OSPF packets.

d.     Only networks configured on the router should be advertised in OSPF, this includes the loopback interface network

e.     Only one router in the internal network should advertise a default route in OSPF

HSRP

1.     Configure HSRP on HQ network

a.     HSRP Requirements

i.     Standby Group ID matches VLAN ID

ii.     Priority:

•     HQ_R1 VLAN 101: 110

•     HQ_R2 VLAN 101: 105

•     HQ_R1 VLAN 201: 105

•     HQ_R2 VLAN 201: 110

•     HQ_R1 VLAN 50: 110

•     HQ_R2 VLAN 50: 105

iii.     Preempt is configured on all end user facing interfaces

iv.     Virtual IP address (VIP) is the first usable IP address of each network

v.     Track the uplink interface to the Edge router

HQ_S1 Requirements

1.     SSH Requirements

a.     Login credentials:

i     User name: admin

ii     Encrypted Password: cisco

iii     Permission level: 15

b.     IP Domain Name: wcc.edu

c.     RSA Key Bit Size: 1024

d.     VTY Access: SSH Only

e.     Authentication Method: Local

2.     Default Gateway: HSRP VIP

3.     Enable Secret: class

Remote DHCP

Note:     HQ User 1, HQ User 2, and Branch User 1 will get their network configuration via DHCP

1.     DHCP Server:

a.     Edge Router is DHCP server

b.     Exclude IPs .0 - .50 for all DHCP networks

c.     DHCP relay IP: Edge loopback IP address

d.     Pool Requirements:

Note: Pool names are case sensitive and must match the pool name in the instructions

i.     Pool HR

•     Name: HR

•     Network: HR VLAN network

•     Default Router: First usable IP address of HR network

ii.     Pool SALES

•     Name: SALES

•     Network: Sales VLAN network

•     Default Router: First usable IP address of Sales network

iii.     Pool R&D

•     Name: R&D

•     Network: R&D VLAN network

•     Default Router: First usable IP address of R&D network

Etherchannel

1.     Configure EtherChannel Between HQ_S1 and HQ_S2

a.     Requirements:

•     Protocol: PAgP

•     Channel Group number: 1

•     Hard coded to be a trunk

•     Native vlan: 99

•     DTP is disabled

Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| ISP | Gig 0/0/0 | 135.84.124.1 | 255.255.255.252 |
| ISP | Gig 0/0 | 8.8.8.1 | 255.255.255.0 |
| Edge | Gig 0/0/0 | 135.84.124.2 | 255.255.255.252 |
| Edge | Gig 0/1/0 | 10.0.0.1 | 255.255.255.252 |
| Edge | Gig 0/2/0 | 10.0.0.5 | 255.255.255.252 |
| Edge | Gig 0/2 | 10.0.0.9 | 255.255.255.252 |
| Edge | Loopback 1 | 10.255.255.1 | 255.255.255.255 |
| HQ_R1 | Gig 0/0/0 | 10.0.0.2 | 255.255.255.252 |
| HQ_R1 | Gig 0/2.101 | 172.26.101.2 | 255.255.255.0 |
| HQ_R1 | Gig 0/2.201 | 172.26.201.2 | 255.255.255.0 |
| HQ_R1 | Gig 0/2.50 | 172.26.50.2 | 255.255.255.0 |
| HQ_R1 | Gig 0/2.99 |  |  |
| HQ_R2 | Gig 0/0/0 | 10.0.0.6 | 255.255.255.252 |
| HQ_R2 | Gig 0/2.101 | 172.26.101.3 | 255.255.255.0 |
| HQ_R2 | Gig 0/2.201 | 172.26.201.3 | 255.255.255.0 |
| HQ_R2 | Gig 0/2.50 | 172.26.50.3 | 255.255.255.0 |
| HQ_R2 | Gig 0/2.99 |  |  |
| Branch_R1 | Gig 0/2 | 10.0.0.10 | 255.255.255.252 |
| Branch_R1 | SVI 11 | 192.168.11.1 | 255.255.255.0 |
| HQ_S1 | SVI 50 | 172.26.50.10 | 255.255.255.0 |

HQ_S1 + SSH + EtherChannel

- host HQ_S1

- vlan 101

  - name HR

- vlan 201

  - name Sales

- vlan 50

  - name MGMT

- vlan 99

  - name native

- int g0/1

  - swi mode trunk

  - swi trunk native vlan 99

  - swi trunk allowed vlan 101,201,50

  - swi non

- int f0/1

  - swi mode acc

  - swi acc vlan 101

  - spanning-tree portfast

  - spanning-tree bpduguard enable

  - swi port-security

  - swi port-security max 3

  - swi port-security mac-address sticky

  - swi port-security violation protect

- int vlan 50

  - ip address 172.26.50.10 255.255.255.0

- ip default-gateway 172.26.50.1

- line con 0

  - enable secret class

  - logging synchronous

- username admin privilege 15 secret cisco

- no ip domain-lookup

- ip domain-name  wcc.edu

- crypto key generate rsa general-keys modulus 1024

- line vty 0 15

  - transport input ssh

  - login local

- int range f0/23-24

  - shut

  - channel-group 1 mode desirable

  - no shut

- int port-c 1

  - swi mode trunk

  - swi non

  - swi trunk nat vlan 99

  - swi trunk allowed vlan 101,201,50

HQ_S2 + EtherChannel

- host HQ_S2

- vlan 101

  - name HR

- vlan 201

  - name Sales

- vlan 50

  - name MGMT

- vlan 99

  - name native

- int g0/1

  - swi mode trunk

  - swi trunk native vlan 99

  - swi trunk allowed vlan 101,201,50

  - swi non

- int f0/1

  - swi mode acc

  - swi acc vlan 201

  - spanning-tree portfast

  - spanning-tree bpduguard enable

  - swi port-security

  - swi port-security max 3

  - swi port-security mac-address sticky

  - swi port-security violation protect

- int range f0/23-24

  - shut

  - channel-group 1 mode desirable

  - no shut

- int port-c 1

  - swi mode trunk

  - swi non

  - swi trunk nat vlan 99

  - swi trunk allowed vlan 101,201,50

Branch_S1

- host Branch_S1

- vlan 11

  - name R&D

- vlan 99

  - name native

- int g0/1

  - swi mode trunk

  - swi trunk native vlan 99

  - swi trunk allowed vlan 11

  - swi non

- int f0/1

  - swi mode acc

  - swi acc vlan 11

  - spanning-tree portfast

  - spanning-tree bpduguard enable

  - swi port-security

  - swi port-security max 3

  - swi port-security mac-address sticky

  - swi port-security violation protect

Branch_R1

- host Branch_R1

- ip routing

- vlan 11

  - name R&D

- vlan 99

  - name native

- interface g0/2

  - no switchport

  - ip address 10.0.0.10 255.255.255.252

  - no shutdown

- interface vlan 11

  - ip address 192.168.11.1 255.255.255.0

  - ip helper-address 10.255.255.1

- int g0/1

- swi trunk encapsulation dot1q (nothing else needed)

- swi mode trunk

- swi trunk native vlan 99

- swi trunk allowed vlan 11

- swi non

HQ_R1 + DHCP Relay + HSRP

- host HQ_R1

- int g0/0/0

  - ip add 10.0.0.2 255.255.255.252

  - no shut

- int g0/2

  - no ip add

  - no shut

- int g0/2.101

  - encap dot 101

  - ip add 172.26.101.2 255.255.255.0

  - ip helper-address 10.255.255.1

  - standby 101 track GigabitEthernet 0/0/0 (ISP/WAN-facing link)

  - standby 101 ip 172.26.101.1

  - standby 101 priority 110

  - standby 101 preempt

- int g0/2.201

  - encap dot 201

  - ip add 172.26.201.2 255.255.255.0

  - ip helper-address 10.255.255.1

  - standby 201 track GigabitEthernet 0/0/0

  - standby 201 ip 172.26.201.1

  - standby 201 priority 105

  - standby 201 preempt

- int g0/2.50

  - encap dot 50

  - ip add 172.26.50.2 255.255.255.0

  - ip helper-address 10.255.255.1

  - standby 50 track GigabitEthernet 0/0/0

  - standby 50 ip 172.26.50.1

  - standby 50 priority 110

  - standby 50 preempt

- int g0/2.99

  - encap dot 99 native

HQ_R2 + DHCP Relay + HSRP

- host HQ_R2

- int g0/0/0

  - ip add 10.0.0.6 255.255.255.252

  - no shut

- int g0/2

  - no ip add

  - no shut

- int g0/2.101

  - encap dot 101

  - ip add 172.26.101.3 255.255.255.0

  - ip helper-address 10.255.255.1

  - standby 101 track GigabitEthernet 0/0/0 (ISP/WAN-facing link)

  - standby 101 ip 172.26.101.1

  - standby 101 priority 105

  - standby 101 preempt

- int g0/2.201

  - encap dot 201

  - ip add 172.26.201.3 255.255.255.0

  - ip helper-address 10.255.255.1

  - standby 201 track GigabitEthernet 0/0/0

  - standby 201 ip 172.26.201.1

  - standby 201 priority 110

  - standby 201 preempt

- int g0/2.50

  - encap dot 50

  - ip add 172.26.50.3 255.255.255.0

  - ip helper-address 10.255.255.1

  - standby 50 track GigabitEthernet 0/0/0

  - standby 50 ip 172.26.50.1

  - standby 50 priority 105

  - standby 50 preempt

- int g0/2.99

  - encap dot 99 native

- exit

ISP + Default Route

- host ISP

- int g0/0/0

  - ip add 135.84.124.1 255.255.255.252

  - no shut

- int g0/0

  - ip add 8.8.8.1 255.255.255.0

  - no shut

- ip route 0.0.0.0 0.0.0.0 135.84.124.2

Edge + Default Route + DHCP Server

- host Edge

- int g0/0/0

  - ip add 135.84.124.2 255.255.255.252

  - no shut

- int g0/1/0

  - ip add 10.0.0.1 255.255.255.252

  - no shut

- int g0/2/0

  - ip add 10.0.0.5 255.255.255.252

  - no shut

- int g0/2

  - ip add 10.0.0.9 255.255.255.252

  - no shut

- int loopback 1

  - ip add 10.255.255.1 255.255.255.255

- ip route 0.0.0.0 0.0.0.0 135.84.124.1

  - ip dhcp excluded-address 172.26.101.0 172.26.101.50

  - p dhcp excluded-address 172.26.201.0 172.26.201.50

  - ip dhcp excluded-address 192.168.11.0 192.168.11.50

  - ip dhcp pool HR

    - network 172.26.101.0  255.255.255.0

    - default-router 172.26.101.1 (network’s default Router ip, not Edge’s)

  - ip dhcp pool SALES

    - network 172.26.201.0 255.255.255.0

    - default-router 172.26.201.1

- ip dhcp pool R&D

  - network 192.168.11.0  255.255.255.0

  - default-router 192.168.11.1

  - do sho run

OSPF

Edge

- ip routing (enables ipv4 routing?)

- router ospf 10

  - router-id 1.1.1.1

  - default-information originate (default route because this is the edge router)

  - passive-interface default

  - no passive-interface g0/0/0 (because this leads to an OSPF neighbor)

  - no passive-interface g0/1/0 (same reason)

  - no passive-interface g0/2/0 (same reason)

  - Networks:

- do show ip route (adverstise all C connections in the table, except external to ISP because that’s the default)

- Example:

- C 10.0.0.0/30 is directly connected, GigabitEthernet1/1/1

- C 10.0.0.4/30 is directly connected, GigabitEthernet1/1/2

- C 10.0.0.8/30 is directly connected, GigabitEthernet1/1/3

- C 10.255.255.1/32 is directly connected, Loopback1

- Edge(config-router)# network 10.0.0.0 0.0.0.3 area 0

- Edge(config-router)# network 10.0.0.4 0.0.0.3 area 0

- Edge(config-router)# network 10.0.0.8 0.0.0.3 area 0

- Edge(config-router)# network 10.255.255.1 0.0.0.0 area 0

- fd

- dfs

- 10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

- C 10.0.0.0/30 is directly connected, GigabitEthernet0/1/0

- L 10.0.0.1/32 is directly connected, GigabitEthernet0/1/0

- C 10.0.0.4/30 is directly connected, GigabitEthernet0/2/0

- L 10.0.0.5/32 is directly connected, GigabitEthernet0/2/0

- C 10.0.0.8/30 is directly connected, GigabitEthernet0/2

- L 10.0.0.9/32 is directly connected, GigabitEthernet0/2

- C 10.255.255.1/32 is directly connected, Loopback1

- 135.84.0.0/16 is variably subnetted, 2 subnets, 2 masks

- C 135.84.124.0/30 is directly connected, GigabitEthernet0/0/0

- L 135.84.124.2/32 is directly connected, GigabitEthernet0/0/0

- S* 0.0.0.0/0 [1/0] via 135.84.124.1

- network 10.0.0.0 0.0.0.3 area 0

- network 10.0.0.4 0.0.0.3 area 0

- network 10.0.0.8 0.0.0.3 area 0

- network 10.255.255.1 0.0.0.0 area 0

- network 135.84.124.0 0.0.0.3 area 0

HQ_R1

- ip routing

- router ospf 10

- router-id 2.2.2.2

- passive-interface default

- no passive-interface g0/0/0 (because this leads to an OSPF neighbor)

- Networks:

- HQ_R2(config-router)# do show ip route (adverstise all C connections in the table, except external to ISP because that’s the default)

- Example:

- 10.0.0.0/30 is subnetted, 1 subnets

- C 10.0.0.4 is directly connected, GigabitEthernet1/1/1

- 172.16.0.0/24 is subnetted, 3 subnets

- C 172.16.10.0 is directly connected, Vlan10

- C 172.16.20.0 is directly connected, Vlan20

- C 172.16.50.0 is directly connected, Vlan50

- HQ_R2(config-router)# network 10.0.0.4 0.0.0.3 area 0

- HQ_R2(config-router)# network 172.16.10.0 0.0.0.255 area 0

- HQ_R2(config-router)# network 172.16.20.0 0.0.0.255 area 0

- HQ_R2(config-router)# network 172.16.50.0 0.0.0.255 area 0

- 10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

- C 10.0.0.0/30 is directly connected, GigabitEthernet0/0/0

- L 10.0.0.2/32 is directly connected, GigabitEthernet0/0/0

- 172.26.0.0/16 is variably subnetted, 6 subnets, 2 masks

- C 172.26.50.0/24 is directly connected, GigabitEthernet0/2.50

- L 172.26.50.2/32 is directly connected, GigabitEthernet0/2.50

- C 172.26.101.0/24 is directly connected, GigabitEthernet0/2.101

- L 172.26.101.2/32 is directly connected, GigabitEthernet0/2.101

- C 172.26.201.0/24 is directly connected, GigabitEthernet0/2.201

- L 172.26.201.2/32 is directly connected, GigabitEthernet0/2.201

- network 10.0.0.0 0.0.0.3 area 0

- network 172.26.50.0 0.0.0.255 area 0

- network 172.26.101.0 0.0.0.255 area 0

- network 172.26.201.0 0.0.0.255 area 0

HQ_R2

- ip routing

- router ospf 10

- router-id 3.3.3.3

- passive-interface default

- no passive-interface g0/0/0 (because this leads to an OSPF neighbor)

- Networks:

- HQ_R2(config-router)# do show ip route (adverstise all C connections in the table, except external to ISP because that’s the default)

- Example:

- 10.0.0.0/30 is subnetted, 1 subnets

- C 10.0.0.4 is directly connected, GigabitEthernet1/1/1

- 172.16.0.0/24 is subnetted, 3 subnets

- C 172.16.10.0 is directly connected, Vlan10

- C 172.16.20.0 is directly connected, Vlan20

- C 172.16.50.0 is directly connected, Vlan50

- HQ_R2(config-router)# network 10.0.0.4 0.0.0.3 area 0

- HQ_R2(config-router)# network 172.16.10.0 0.0.0.255 area 0

- HQ_R2(config-router)# network 172.16.20.0 0.0.0.255 area 0

- HQ_R2(config-router)# network 172.16.50.0 0.0.0.255 area 0

- 10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

- C 10.0.0.4/30 is directly connected, GigabitEthernet0/0/0

- L 10.0.0.6/32 is directly connected, GigabitEthernet0/0/0

- 172.26.0.0/16 is variably subnetted, 6 subnets, 2 masks

- C 172.26.50.0/24 is directly connected, GigabitEthernet0/2.50

- L 172.26.50.3/32 is directly connected, GigabitEthernet0/2.50

- C 172.26.101.0/24 is directly connected, GigabitEthernet0/2.101

- L 172.26.101.3/32 is directly connected, GigabitEthernet0/2.101

- C 172.26.201.0/24 is directly connected, GigabitEthernet0/2.201

- L 172.26.201.3/32 is directly connected, GigabitEthernet0/2.201

- network 10.0.0.4 0.0.0.3 area 0

- network 172.26.50.0 0.0.0.255 area 0

- network 172.26.101.0 0.0.0.255 area 0

- network 172.26.201.0 0.0.0.255 area 0

Branch_R1

- ip routing

- router ospf 10

- router-id 4.4.4.4

- passive-interface default

- no passive-interface g0/2 (because this leads to an OSPF neighbor)

- Networks:

  - HQ_R2(config-router)# do show ip route (adverstise all C connections in the table, except external to ISP because that’s the default)

  - Example:

  - 10.0.0.0/30 is subnetted, 1 subnets

  - C 10.0.0.4 is directly connected, GigabitEthernet1/1/1

  - 172.16.0.0/24 is subnetted, 3 subnets

  - C 172.16.10.0 is directly connected, Vlan10

  - C 172.16.20.0 is directly connected, Vlan20

  - C 172.16.50.0 is directly connected, Vlan50

  - HQ_R2(config-router)# network 10.0.0.4 0.0.0.3 area 0

  - HQ_R2(config-router)# network 172.16.10.0 0.0.0.255 area 0

  - HQ_R2(config-router)# network 172.16.20.0 0.0.0.255 area 0

  - HQ_R2(config-router)# network 172.16.50.0 0.0.0.255 area 0

- 10.0.0.0/30 is subnetted, 1 subnets

- C 10.0.0.8 is directly connected, GigabitEthernet0/2

- C 192.168.11.0/24 is directly connected, Vlan11

- n

- network 10.0.0.8 0.0.0.3 area 0

- network 192.168.11.0 0.0.0.255 area 0

Connectivity

1.     End user devices get IP via DHCP

2.     End user devices can ping each other, and NotGoogle server (8.8.8.8).

3.     All end user devices can SSH into HQ_S1